# TechNova Enterprise Network — Project Documentation

## 1. Project Overview

Project: TechNova Enterprise Network Infrastructure
Platform: Cisco Packet Tracer
Architecture: Enterprise hierarchical network

The network was designed to provide segmentation, redundancy, dynamic routing, secure management, Internet access, and centralized network services for a medium-sized organization.

### Main technologies implemented

- VLANs and 802.1Q trunking

- Inter-VLAN routing

- HSRP

- EtherChannel/LACP

- Rapid-PVST

- OSPF

- NAT/PAT

- DHCP and DHCP Relay

- DNS

- SSH

- TACACS+

- ACL

- Port Security

- DHCP Snooping

- BPDU Guard

- Root Guard

- Loop Guard

- Router Loopback management

# 2. Network Devices

| Layer/Role | Devices |
| --- | --- |
| Internet/Edge | ISP-R1, EDGE-R1 |
| Core | CORE-R1 |
| Distribution | DIST1, DIST2 |
| Access | Access_SW_1, Access_SW_2, Access_SW_3, Access_SW_4 |
| Infrastructure | Server_SW, MGMT_SW |
| Servers | DHCP, DNS, File, AAA/TACACS+, Public Web |

The network follows a simplified Core → Distribution → Access architecture.

# 3. VLAN & IP Addressing

| VLAN | Name | Network | Purpose |
| --- | --- | --- | --- |
| 10 | HR | 10.10.10.0/24 | Human Resources |
| 20 | IT | 10.10.20.0/24 | IT Department |
| 30 | FINANCE | 10.10.30.0/24 | Finance |
| 40 | GUEST | 10.10.40.0/24 | Guest users |
| 50 | SERVERS | 10.10.50.0/24 | Internal servers |
| 60 | MANAGEMENT | 10.10.60.0/24 | Network management |
| 999 | UNUSED-NATIVE | Reserved | Native/unused VLAN |

VLAN 999 is used as the native VLAN on trunk links instead of VLAN 1.

# 4. Management Addressing

Switches use VLAN 60 for management:

| Device | SSH IP |
| --- | --- |
| DIST1 | 10.10.60.2 |
| DIST2 | 10.10.60.3 |
| Access_SW_1 | 10.10.60.10 |
| Access_SW_2 | 10.10.60.15 |
| Access_SW_3 | 10.10.60.30 |
| Access_SW_4 | 10.10.60.40 |
| MGMT_SW | 10.10.60.50 |
| Server_SW | 10.10.60.60 |

Routers use Loopback interfaces:

| Router | Loopback / SSH IP |
| --- | --- |
| CORE-R1 | 10.255.255.1/32 |
| EDGE-R1 | 10.255.255.2/32 |

The router Loopbacks are advertised through OSPF for reliable in-band management.

# 5. Layer 2 Design

### VLANs & Trunking

Access ports are assigned to appropriate departmental VLANs, while inter-switch connections use 802.1Q trunks.

Trunks use:

Native VLAN: 999

Only required VLANs are permitted across trunks where appropriate.

### EtherChannel

LACP EtherChannels were implemented on several redundant/high-bandwidth links.

The working EtherChannels include:

- DIST1 ↔ DIST2

- DIST1 ↔ Access_SW_1

- DIST1 ↔ Access_SW_2

- DIST2 ↔ Access_SW_3

- DIST2 ↔ infrastructure switches

The DIST2 ↔ Access_SW_4 connection was ultimately left as a normal trunk link because the Packet Tracer EtherChannel implementation repeatedly caused instability.

# 6. Spanning Tree

The network uses:

Rapid-PVST

STP root responsibilities are distributed between the two distribution switches.

DIST1 is preferred root for selected VLANs, while DIST2 is preferred root for other VLANs.

This provides better traffic distribution instead of forcing all VLANs through one distribution switch.

Additional STP protections implemented include:

- PortFast

- BPDU Guard

- Root Guard

- Loop Guard

These protect against accidental loops, unauthorized switches, and unexpected BPDUs.

# 7. First-Hop Redundancy

HSRP is implemented between DIST1 and DIST2.

The distribution switches provide redundant default gateways for the VLANs.

The HSRP model uses:

Virtual Gateway: .1

DIST1:           .2

DIST2:           .3

For example:

VLAN 10

Virtual Gateway → 10.10.10.1

DIST1            → 10.10.10.2

DIST2            → 10.10.10.3

If the active HSRP distribution switch becomes unavailable, the standby switch can take over the virtual gateway.

# 8. Layer 3 Routing

### OSPF

OSPF Area 0 is used as the internal dynamic routing protocol.

Routing relationships exist between:

EDGE-R1

↓

CORE-R1

↓

DIST1 / DIST2

OSPF provides dynamic route exchange and eliminates the need for manually configured routes throughout the internal network.

The EDGE router originates a default route into OSPF so internal devices can reach external networks.

# 9. Internet Connectivity

The Internet path is:

Internal LAN

↓

DIST1 / DIST2

↓

CORE-R1

↓

EDGE-R1

↓

ISP-R1

↓

Internet

EDGE-R1 performs PAT/NAT overload for internal private networks.

The public web server is placed on the ISP/outside network:

198.51.100.0/24

Web server:

198.51.100.10

This allows the public server to be treated as an externally reachable service rather than an internal server requiring inbound NAT.

Internet connectivity was successfully tested using DNS resolution and web access.

# 10. Network Services

### DHCP

A centralized DHCP server is located in the server network:

DHCP Server: 10.10.50.10

DHCP Relay (ip helper-address) is used so clients in different VLANs can obtain addresses from the centralized DHCP server.

### DNS

DNS Server: 10.10.50.11

DNS provides name resolution for internal/external resources.

### File Server

File Server: 10.10.50.12

The server resides in VLAN 50.

### AAA/TACACS+

AAA Server: 10.10.60.20

TACACS+ provides centralized authentication for network-device management.

# 11. Secure Device Management

Network devices are managed using SSH instead of Telnet.

TACACS+ is used for centralized authentication with local authentication configured as a fallback.

Typical design:

Management PC

↓

SSH

↓

Network Device

↓

TACACS+

↓

AAA Server

Local credentials provide fallback access when the TACACS+ server is unavailable.

The console is kept independent from centralized SSH authentication to avoid losing local console access during AAA problems.

# 12. Layer 2 Security

Several security controls were implemented.

### Port Security

End-device ports use:

- Maximum MAC addresses

- Sticky MAC learning

- Shutdown violation mode

- Inactivity aging where appropriate

### DHCP Snooping

DHCP Snooping is enabled on user VLANs.

Trusted ports are configured toward legitimate DHCP/server paths, while client-facing ports remain untrusted.

### STP Protection

Implemented protections include:

- BPDU Guard — protects PortFast access ports from rogue BPDUs

- Root Guard — prevents downstream switches from becoming STP root

- Loop Guard — protects against certain STP failure scenarios

# 13. ACL

Extended ACLs were designed to control communication between departmental VLANs.

For example, the HR security policy was intended to prevent direct HR-to-IT communication while allowing HR traffic toward other permitted destinations.

ACL behavior in Packet Tracer showed some limitations during testing, so this portion was not treated as a fully validated production implementation.

# 14. Redundancy Design

Redundancy exists at multiple levels:

Gateway redundancy

DIST1 ↔ DIST2

HSRP

Link redundancy

Multiple physical links

↓

EtherChannel

STP redundancy

Multiple Layer-2 paths

↓

Rapid-PVST

Routing redundancy

Dynamic OSPF routing

This allows the network to remain operational when individual links or devices fail.

# 15. Troubleshooting & Validation

During implementation, several real configuration and operational issues were encountered and resolved, including:

- VLAN/trunk connectivity problems

- HSRP gateway issues

- DHCP relay problems

- NAT/PAT verification

- SSH connectivity

- Missing RSA keys

- TACACS+ authentication behavior

- STP root selection

- EtherChannel compatibility

- Native VLAN mismatches

- Suspended EtherChannel members

- Packet Tracer state/configuration issues

Common verification commands used included:

show vlan brief

show interfaces trunk

show etherchannel summary

show spanning-tree

show standby brief

show ip route

show ip ospf neighbor

show ip nat translations

show ip interface brief

show running-config

# 16. Final Project Outcome

The completed TechNova network demonstrates the design and implementation of a realistic enterprise network infrastructure rather than a basic connectivity lab.

It combines:

Segmentation + Routing + Redundancy + Security + Network Services + Secure Management + Internet Connectivity

The project demonstrates practical knowledge of technologies commonly encountered in NOC, Network Engineer, and junior network/security roles.

### Technology Summary

Cisco Packet Tracer · VLAN · 802.1Q · STP/Rapid-PVST · EtherChannel/LACP · HSRP · OSPF · NAT/PAT · DHCP · DNS · ACL · SSH · TACACS+ · Port Security · DHCP Snooping · BPDU Guard · Root Guard · Loop Guard